EXP 1 :Develop a Python-based Reinforcement Learning model using the OpenAI Gym environment to solve a Grid World navigation problem where the agent must reach the goal while avoiding obstacles. Explain how the state space, action space, reward function, and constraints are defined.

CODE :

import random

GRID_SIZE = 5

START = (0, 0)

GOAL = (4, 4)

OBSTACLES = [(1, 1), (2, 2), (3, 1)]

ACTIONS = {

0: (-1, 0), # Up

1: (1, 0), # Down

2: (0, -1), # Left

3: (0, 1) # Right

}

state = START

print("Starting Grid World Navigation")

while state != GOAL:

print("Current Position:", state)

action = random.randint(0, 3)

move = ACTIONS[action]

new_row = state[0] + move[0]

new_col = state[1] + move[1]

if new_row < 0 or new_row >= GRID_SIZE or new_col < 0 or new_col >= GRID_SIZE:  reward = -5

print("Moved Outside Grid")

print("Reward:", reward)

elif (new_row, new_col) in OBSTACLES:

reward = -10

print("Hit Obstacle")

print("Reward:", reward)

else:

state = (new_row, new_col)

if state == GOAL:

reward = 100

print("Goal Reached")

print("Reward:", reward)

break

else:

reward = -1

print("Moved to:", state)

print("Reward:", reward)

print("Navigation Completed")

OUTPUT :

EXP 2 : Implement an ε-greedy Multi-Armed Bandit algorithm in Python to maximize rewards under a limited budget of 500 trials. Compare the effect of different ε values (0.1, 0.3, and 0.5) on exploration, exploitation, and constraint satisfaction.

CODE :

import random

# True reward probabilities for 5 arms

true_rewards = [0.2, 0.4, 0.6, 0.8, 0.5]

trials = 500

epsilons = [0.1, 0.3, 0.5]

for epsilon in epsilons:

estimates = [0] * len(true_rewards) # Estimated rewards

counts = [0] * len(true_rewards) # Number of times each arm is selected  total_reward = 0

explore = 0

exploit = 0

for _ in range(trials):

# Exploration or Exploitation

if random.random() < epsilon:

arm = random.randint(0, len(true_rewards) - 1)

explore += 1

else:

arm = estimates.index(max(estimates))

exploit += 1

# Get reward (1 = success, 0 = failure)

reward = 1 if random.random() < true_rewards[arm] else 0

total_reward += reward

counts[arm] += 1

# Update estimated reward

estimates[arm] += (reward - estimates[arm]) / counts[arm]

print("\nε =", epsilon)

print("Total Reward :", total_reward)

print("Average Reward :", round(total_reward / trials, 3))  print("Explorations :", explore)

print("Exploitations :", exploit)

print("Best Arm Selected :", estimates.index(max(estimates)) + 1) OUTPUT :

EXP 3: Design a Markov Decision Process (MDP) for an autonomous robot that must reach a destination while minimizing energy consumption. Define the states, actions, transition probabilities, rewards, and energy constraints.

CODE :

import random

# Grid size

GRID_SIZE = 4

# Start and Goal

start = (0, 0)

goal = (3, 3)

# Energy available

energy = 10

# Actions

actions = {

"UP": (-1, 0),

"DOWN": (1, 0),

"LEFT": (0, -1),

"RIGHT": (0, 1)

}

# Transition probabilities

# 80% move correctly, 20% stay in same place

success_probability = 0.8

state = start

total_reward = 0

print("Autonomous Robot MDP Simulation")

print("Start State:", state)

print("Goal State:", goal)

print("Initial Energy:", energy)

while state != goal and energy > 0:

action = random.choice(list(actions.keys()))

print("\nChosen Action:", action)

if random.random() < success_probability:

move = actions[action]

new_row = state[0] + move[0]

new_col = state[1] + move[1]

if 0 <= new_row < GRID_SIZE and 0 <= new_col < GRID_SIZE:  state = (new_row, new_col)

energy -= 1

if state == goal:

reward = 100

else:

reward = -1

total_reward += reward

print("Current State:", state)

print("Reward:", reward)

print("Remaining Energy:", energy)

if state == goal:

print("\nRobot reached the destination!")

else:

print("\nRobot ran out of energy!")

print("Total Reward:", total_reward)

OUTPUT :

EXP 4 : Using Python and TensorFlow/Keras, implement a value iteration algorithm to determine the optimal policy for a constrained grid environment where certain states are restricted. Explain how Bellman equations are used to obtain the optimal policy.

CODE :

import numpy as np

grid_size = 4

gamma = 0.9

goal = (3, 3)

restricted = [(1, 1), (2, 2)]

values = np.zeros((grid_size, grid_size))

actions = [(-1,0),(1,0),(0,-1),(0,1)]

while True:

delta = 0

new_values = values.copy()

for i in range(grid_size):

for j in range(grid_size):

if (i, j) == goal:

continue

if (i, j) in restricted:

continue

best = -1000

for action in actions:

ni = i + action[0]

nj = j + action[1]

if ni < 0 or ni >= grid_size or nj < 0 or nj >= grid_size:

ni = i

nj = j

if (ni, nj) in restricted:

ni = i

nj = j

reward = -1

if (ni, nj) == goal:

reward = 10

value = reward + gamma * values[ni][nj]

if value > best:

best = value

new_values[i][j] = best

delta = max(delta, abs(values[i][j] - best))

values = new_values

if delta < 0.001:

break

print("Optimal State Values")

print(values)

print("\nOptimal Policy")

policy = [["" for j in range(grid_size)] for i in range(grid_size)] for i in range(grid_size):

for j in range(grid_size):

if (i, j) == goal:

policy[i][j] = "G"

continue

if (i, j) in restricted:

policy[i][j] = "X"

continue

best = -1000

move = ""

directions = {

"U":(-1,0),

"D":(1,0),

"L":(0,-1),

"R":(0,1)

}

for name, action in directions.items():

ni = i + action[0]

nj = j + action[1]

if ni < 0 or ni >= grid_size or nj < 0 or nj >= grid_size:

ni = i

nj = j

if (ni, nj) in restricted:

ni = i

nj = j

reward = -1

if (ni, nj) == goal:

reward = 10

value = reward + gamma * values[ni][nj]

if value > best:

best = value

move = name

policy[i][j] = move

for row in policy:

print(row)

OUTPUT :

EXP 5 : A warehouse robot has a battery capacity of only 30 minutes. Design an RL framework that enables the robot to complete the maximum number of deliveries without exceeding the battery limit. Explain the constraints, policy, and reward design.

CODE :

import random

battery = 30

deliveries = 0

total_reward = 0

actions = ["Deliver", "Wait"]

print("Warehouse Robot Simulation")

print("Battery:", battery, "minutes")

while battery > 0:

action = random.choice(actions)

if action == "Deliver":

time_used = random.randint(3, 6)

if battery >= time_used:

battery -= time_used

deliveries += 1

total_reward += 10

print("Action : Deliver")

print("Time Used :", time_used)

print("Battery Left :", battery)

print("Reward : +10\n")

else:

print("Not enough battery for delivery")

break

else:

battery -= 1

total_reward -= 1

print("Action : Wait")

print("Battery Left :", battery)

print("Reward : -1\n")

print("Simulation Completed")

print("Total Deliveries :", deliveries)

print("Total Reward :", total_reward)

print("Battery Remaining :", battery)

OUTPUT :